PYNQ Delivery Guide

Overlay: matmul_overlay | IP: matmul_accel_0

This document is the handoff note for the PYNQ-side teammate. It consolidates the final overlay file set, the exact IP instance name, the software-visible register map, and the minimum bring-up procedure needed to run the INT8 matrix multiplication accelerator.

# 1. Delivery Files

| File | Required | Purpose |
| --- | --- | --- |
| matmul_overlay.bit | Yes | Overlay bitstream to load on the board |
| matmul_overlay.hwh | Yes | Hardware handoff file that PYNQ parses with the bitstream |
| register_map.actual.json | Yes | Register offsets for RegisterMap.from_json |
| PYNQ_Delivery_Guide.docx | Yes | Handoff guide for the software side |
| matmul_overlay.xsa | No | Reference hardware platform export |
| xmatmul_accel_hw.h | No | Original HLS-generated register definition header |
| csynth.rpt | No | HLS synthesis summary for reference |
| overlay_metadata.json | No | Machine-readable overlay metadata and checksums |

Keep matmul_overlay.bit and matmul_overlay.hwh in the same directory and keep the same base name. This is the most important packaging rule for PYNQ Overlay loading.

# 2. Hardware Summary

| Item | Value |
| --- | --- |
| Overlay base name | matmul_overlay |
| IP instance name | matmul_accel_0 |
| Component name | design_1_matmul_accel_0_0 |
| VLNV | xilinx.com:hls:matmul_accel:1.0 |
| AXI Lite base address | 0x40000000 |
| AXI Lite high address | 0x4000FFFF |
| WNS | 2.58 ns |
| WHS | 0.022 ns |

• The overlay uses AXI Lite only for control and parameter programming.

• The accelerator reads input A and input B from DDR and writes output C back to DDR through m_axi ports.

• No separate CDMA stage is required in the software flow.

• The matmul_overlay.* files are byte-identical to design_1_wrapper.* and were renamed only to provide a clean overlay base name.

# 3. Data Contract

• Input A: int8[M, K], row-major.

• Input B: int8[K, N], row-major.

• Output C: int32[M, N], row-major.

• Kernel equation: C = (A @ B) >> shift.

• Software must pass physically contiguous buffers so the IP can access them through DDR addresses.

# 4. AXI Lite Register Map

| Register | Offset | Meaning |
| --- | --- | --- |
| CTRL | 0x00 | Control register |
| GIE | 0x04 | Global interrupt enable |
| IER | 0x08 | IP interrupt enable |
| ISR | 0x0c | IP interrupt status |
| A_ADDR | 0x10 | Input A DDR address low 32 bits |
| A_ADDR_HIGH | 0x14 | Input A DDR address high 32 bits |
| B_ADDR | 0x1c | Input B DDR address low 32 bits |
| B_ADDR_HIGH | 0x20 | Input B DDR address high 32 bits |
| C_ADDR | 0x28 | Output C DDR address low 32 bits |
| C_ADDR_HIGH | 0x2c | Output C DDR address high 32 bits |
| M | 0x34 | Rows of A and C |
| K | 0x3c | Shared inner dimension |
| N | 0x44 | Columns of B and C |
| SHIFT | 0x4c | Right shift applied after accumulation |

The 64-bit DDR pointers are split into low-word and high-word registers. On the current PYNQ path, the driver should write both halves for A_ADDR, B_ADDR, and C_ADDR before asserting ap_start.

# 5. Basic Bring-up Flow

• Copy matmul_overlay.bit, matmul_overlay.hwh, and register_map.actual.json to the PYNQ workspace.

• Load the overlay with bitfile matmul_overlay.bit.

• Use ip_name matmul_accel_0 when creating the accelerator object.

• Allocate contiguous buffers for A, B, and C, then write their physical addresses into the AXI Lite registers.

• Write M, K, N, and SHIFT, then write 0x01 to CTRL to start the kernel.

• Poll CTRL until bit 1 ap_done is set, then invalidate the output buffer and read back C.

# 6. Python Example

from pynq_driver.matmul_accel import MatmulAccel
from pynq_driver.register_map import RegisterMap

register_map = RegisterMap.from_json("register_map.actual.json")
accel = MatmulAccel(
    bitfile="matmul_overlay.bit",
    ip_name="matmul_accel_0",
    register_map=register_map,
)

output = accel.run(input_a, input_b, shift=0)

# 7. Delivery Notes

• The provided register_map.actual.json includes the interrupt registers as well as the address high-word registers.

• If the software stack prints overlay.ip_dict, the expected kernel entry should be matmul_accel_0.

• For traceability, overlay_metadata.json records the SHA-256 values showing that matmul_overlay.bit/.hwh are identical to the exported design_1_wrapper files.